## “KAVALIER 800”

Designed by Albert Nazarov, Maxim Kovalyov, Dmitry Dolinsky

Published in “Power & Sail Boats” magazine:

Nazarov A., Kovalyov M., Dolinsky "T800 by own hands" // "Power & Sail Boats", St.Petersburg, 2003, No.1 (188) – p.122-125. and No.2 (189) – p.122-125. (in Russian).

Main particulars

| - LOA, m | - 8.00 |
| --- | --- |
| - LWL, m | - 8.00 |
| - Beam, m | - 2.50 |
| - Draught, m | - 0.63 / 2.00 |
| - Empty weight, DSPL, kg | - 1200 |
| - Sail Area, SA, m2: |  |
| - - main | - 30.0 |
| - - jib | - 13.5 |
| - - gennaker | - 38 |
| - Dimensionless parameters: |  |
| - L/DSPL1/3 | - 7.52 |
| - SA1/2/DSPL1/3 | - 6.21 |
| - Prismatic coefficient, CP | - 0.57 |
| - LCB, % | - 3.5 |

GENERAL ARRANGEMENT:

1- forepeak; 2 – double berth; 3 – hanging locker; 4 - head; 5 - galley; 6 - sofa; 7 - seat; 8 – chart table; 9 - berth; 10 - locker; 11 – gas bottle locker; 12 – outboard motor compartment; 13 - table; 14 - cockpit; 15 - pillar

SAIL PLAN:

Spar: mast section - 130802.5 mm, top 100802.5 mm, aluminum alloy; boom - pipe 1202 mm, aluminum alloy; bowsprit - pipe 802 mm, aluminum alloy; spreaders - pipe 283, aluminum alloy; Standing rigging: lower shrouds - cable 119, d=5; intermediate shrouds - cable 119, d=4; upper shrouds - cable 119, d=4; forestay - cable 119, d=5; afterstay - cable 119, d=4; runners - cable 119, d=4;